PRODUCTION EQUIPMENT DIAGNOSTICS VIEWED AS BASIS

FOR INVESTMENTS

#### Novica Grujic, Drago Soldat*

Technical College, 12000 Pozarevac, Yugoslavia

* Institute of Investment Economics, 11000 Belgrade, Yugoslavia

###### ABSTRACT

Application of various methods of production equipment diagnostics makes it possible to determine weak spots and possible breakdowns. Knowing weak spots and possible breakdowns forms the basis for making decisions on the need to invest. The achievement of set objectives is related to the process of production equipment exploitation. State-dependent maintenance performed by qualified staff gives satisfactory results.

##### 1. INTRODUCTION

State dependent maintenance which is a specific method of preventive maintenance is especially important in the process of production equipment diagnostics during its exploitation. The objective of diagnostics is determination of weak spots and possible breakdowns which form the basis for making decisions on whether to undertake investment activities or not.

Production equipment diagnostics (aimed at prediction of weak spots and determination of possible breakdowns) should be prepared during its development through design and construction phases based on exploitation data which stems from state dependent maintenance activities. This actually means that during the period of exploitation of production equipment maintenance staff has the greatest responsibility for the success of the job in question.

Various diagnostic methods are applied to those parts (elements) of production equipment which represent possible “weak spots” and they are aimed at product development and advancement.

In this paper, diagnostics is based on monitoring mechanical production equipment in operation. This does not, however, diminish significance of other types of equipment which are not included in this discussion. Continuous monitoring of selected parameters of production equipment is performed by supervision of its current condition. Preventive maintenance measures are used to keep equipment in its operational state.

State-dependent maintenance plays a significant role in the area of diagnostics because elimination of weak spots and prevention of unexpected breakdowns delays the need for investments during the economic life of a project.

Achievement of set goals requires adequately trained maintenance personnel.

2. CHARACTERISTICS OF STATE DEPENDENT MAINTENANCE

In order to bring production equipment into such a state which would make production objectives attainable, appropriate maintenance methods are applied. There are two basic maintenance methods: corrective and preventive maintenance.

Objectively speaking, it is difficult to eliminate corrective maintenance entirely, but it can be minimized by appropriate organization and by application of preventive maintenance methods. State dependent maintenance is especially significant because it forms part of the diagnostic process whose aim is to define the state of each piece of production equipment by monitoring it during operation.

State dependent maintenance practically monitors the “health” of production equipment and its components, just like a physician monitors the health of a patient. This is done in order to undertake necessary activities such as planned replacements or repairs of component parts of production equipment all with the aim of elimination of delays. This maintenance method can be applied to production equipment for which it is possible to perform diagnostics of component parts.

State dependent maintenance method can be realized through following models:

- Reliability level control,

- State parameters control.

State dependent maintenance by reliability level control is based on unlimited operation of production equipment; necessary maintenance activities are performed when breakdown occurs, while the actual reliability level is within prescribed values. Once the prescribed reliability level is exceeded, causes of deviations are analyzed in order to take measures to raise reliability to the necessary (prescribed) level.

State dependent maintenance by state parameters control is based on the periodic or continuous monitoring of selected parameters of the state in which the production equipment is, in order to detect pre-breakdown state and to monitor the wear level. Various parameters can be used to determine the current state: noise, vibrations, wear, fatigue, worn-out component parts, etc. These parameters can be determined for any component of production equipment depending on its function and requirements, working methods, working conditions etc. 1.

3. DETECTION AND ANALYSIS OF WEAK SPOTS AND BREAKDOWNS

Weak or critical spots are those where delays caused by breakdown of individual components of production equipment are more frequent than usually. They are detected by the application of appropriate diagnostic methods. The easiest way to detect a weak spot is to monitor breakdowns by utilization of appropriate diagnostic methods. During this process, causes are determined and appropriate solutions and improvements are devised 2. If certain production equipment components wear out normally in expected time intervals under normal operating conditions they cannot be classified as weak spots. Weak spots occur due to:

- Bad design solutions,

- Unsatisfactory manufacturing methods,

- Inappropriate handling and maintenance,

- Inappropriate work process,

- External influences,

- Inappropriate material, etc.

Diagnostic methods are applied to research weak spots on the reducing gear of the bogie wheel of a rotary excavator. Research described in this work shows that the actual weak spots are buckets on the bogie wheel and the reducing gear of the bogie wheel 3.

Table 1.

Mathematical model of the reducing gear of the bogie wheel of a rotary excavator.

| Diagnostic parameter | Label | Boundary values of parameters | Boundary | e0 | e1 | e2 | e3 | E4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Oil pressure | P | 0.8 bar 4.0 bar | min max | 1 | 0 | 1 | 1 | 1 |
| Torque | M | 2.7 KNm | max | 1 | 1 | 0 | 1 | 1 |
| Bearing temperature | T | 800C | max | 1 | 1 | 1 | 0 | 1 |
| Oilflow | Q | 0.9 lit/min | min | 1 | 1 | 1 | 1 | 0 |

The model is based on the use of functional diagnostics with two basic states “functioning” and “malfunctioning” and its function is to control the system and protect it from breakdowns. According to these objectives, the mathematical model of the reducing gear can be represented by the table of functions (state ei , i = 0,1,2,3,4: “1” - functioning, “0” - malfunctioning) as in Table 1.

Current state of the permanent reducing gear control is based on:

- Control of the oil pressure in the lubricating installation (minimum pressure 0.8 bar and maximum 4 bar, i.e. 0.8  p  4bar),

- The use of the switch in the oil installation which reacts as pressure exceeds stated values and interrupts the circuit,

- Torque control by the safety friction coupling which yields maximum torque value of 2.7 KNm.

Analyses of reducing gear breakdowns show that bearings on the input shaft are one of the weak spots. In order to reduce the influence of weak spots in this particular case, it is necessary to perform permanent control of input shaft bearing temperature and oil flow.

Forecasting of possible breakdowns in production equipment is closely connected to detection and detailed analysis of the causes of “weak spots”, considering the fact that elimination of breakdowns considerably decreases the number of delays, which positively affects the efficiency of maintenance 2.

It is important to be able to predict possible breakdowns, their location and frequency and to estimate the time required to fix the breakdown and to revert the technical system into its functional state.

The first approach is based on ranking by importance the breakdowns most likely to occur in various components subjected to diagnostics.

The second approach is oriented exclusively on experiments and intuition of the designer and it does not take into account the probability of breakdowns. According to this approach, in certain cases the list of breakdowns is compiled based on experiments, for example in breakdowns described as permanent malfunctionings. In other cases the list is determined according to selected diagnostic methods and tools.

In such cases the designer of the diagnostic system checks the functioning of the device, the scope of analysis and diagnostic time.

When selecting diagnostic methods and tools certain factors such as the specifics of production, operating regimes and servicing of the subject being diagnosed have to be taken into account. It is convenient to represent diagnostic systems for complex subjects according to hierarchical principle with local and general means of functional and test diagnostics.

Continuous monitoring of selected parameters of production equipment actually represents supervision which illustrates its current state. The aim of preventive maintenance is to keep the current state functional. The basic objective of supervision of production equipment is continuous monitoring of input and output values 4.

The change in the output signal Y(t) can result from the change in the internal parameter  or state variable X. This is shown in Fig.1.

Fig.1. Diagram of a technical system with varying influences on its operation.

Deviations from regular state can be caused not only by weak spots on production equipment but also by errors of different origin. If errors of any type are not eliminated in time, this can cause repeated interruptions and eventual breakdown of production equipment. Fig.2. describes behavior of production equipment caused by an error related to certain interruptions, i.e. its breakdown in exploitation stage, while Fig.3. gives a representation of operational capability, with and without breakdowns, after which certain components of production equipment must be renewed.

Fig.2. Production equipment behavior caused by error.

Fig.3. Functionality and reliability of technical systems:

a) without breakdowns,	b) after repairs.

4. DIAGNOSTICS METHODS

4.1. Functional diagnostics method.

Functional diagnostics consists of control of production equipment's parameters of state (temperature, pressure, flow, etc.) by monitoring and collecting data about changes. This can be done efficiently with the support of an appropriate software package 1.

Diagram of functional diagnostics is given in Fig.4, in which actions dj represent operating influences, when the object in question is used for its intended use or is in the regime which simulates operation. Diagnostic tools are used to measure control signals yj and Rj* signals which represent the response of the object to actions dj.

Fig.4. Diagram of the functional diagnostics system.

Signals yj are required in cases where it is necessary to control the physical model and control block depending on the operating regime of the object. The control block uses signals yj as well as feedback signals obtained from results decoding block in order to establish commuted connection.

If protection function exists within the functional diagnostics system, the results decoding block (RDB) issues commands for control of the object. This block compares the actual results of elementary tests Rj* with possible results Rj, Rji obtained from the physical model. When analyzing the problem of proper functioning of the object, physical model stores and feeds results only to set Rj 3.

The functional diagnostics system represents a widely used system for centralized control. According to this system, technical state of the object is determined based on the results obtained by comparison of actual values of parameters with their highest acceptable values. In these cases the physical model of the object being diagnosed can be represented by the device used for storing and feeding known acceptable values of controlled parameters.

The designer of diagnostic systems needs to know whether functional diagnostics system is necessary for the object being diagnosed. Possible answers to this question are:

- The functional diagnostics system is unnecessary when the value of probability P(t) of uninterrupted operation of the object being diagnosed, after a long period of continuous operation, does not fall under the acceptable level, i.e. under the acceptable value of the probability of proper functioning of the object being diagnosed.

- The functional diagnostics system is necessary in cases where high probability of proper functioning of the object being diagnosed has to be achieved.

In functional diagnostics, the scope of breakdown examination is defined after the definition of design conditions, depending on the accuracy with which it is required to know the spot where breakdown occurs.

Each such spot is equipped with its own tools for functional diagnostics with output signals which directly or after logical processsing point to the defective spot. In functional diagnostics systems, signals represent responses to the state of the object and show that automatic processing of statistical data on breakdowns gives a picture of the system and the state it is in during exploitation.

4.2. Test diagnostics method.

While functional diagnostics represents control of state parameters of production equipment based on their permanent changes, test diagnostics represents periodic control of parameters which describe the state of the subject examined. Here too, it is possible to increase efficiency by the application of an appropriate software package 1.

Unlike functional diagnostics, test diagnostics is always necessary. After the test diagnostics system is created, it is necessary to check the functioning of the object being diagnosed and tools for its functional diagnostics. When designing the test diagnostics system the possibilities for detection and analysis of breakdowns should be provided for. The selection of tools for test diagnostics depends upon the nature of possible breakdowns. The number of possible breakdowns should also include breakdowns of the spare device which serves as replacement for the one being diagnosed.

Characteristic feature of the test diagnostics system given in Fig.5. is that the control block (which contains the algorithm for diagnosis) generates actions dj of elementary tests tj  T and sends them via the communication system to the object being diagnosed. The source of actions can be used to influence the physical model of the object as well.

Fig.5. Diagram of the test diagnostics system.

Control block can exert direct influence on the object via the communication device. In general case, the communication system can perform commutation of the communication channel depending on the signals received from the control block. When this system is used to verify the functioning of the object, the physical model is represented by the function 1,3:

Rj = (tj), for tj  T.

Together with the output of the physical model a set of signals Rj is given. When the state of the object is known, then the physical model has to contain relationships given above as well as the following:

Rji = i(tj), for ni  N and tj  T,

i.e. set of output signals of the physical model forms the set Rj or Rji.

Object which is functioning properly produces the set of signals Rj, while the one which is malfunctioning produces the set Rji.

Physical model provides information about possible technical states of the object represented by possible results Rj and Rji obtained by elementary tests from set T. Responses of the object to actions dj are represented by the results Rj* of elementary tests tj  T.

The next elementary test is determined according to actual results Rj* obtained by previous elementary tests. Results decoding block compares possible Rj and Rji with actual results of elementary tests Rj*, determines future elementary tests and forms results 1.

Processing of data about state parameters obtained according to a suitable model, enables us not only to forecast breakdowns but also to calculate reliability.

4.3. Diagnostics of roller bearings in belt conveyors.

Previously described theoretical considerations of functional and test diagnostics of production equipment have been applied in this paper to the diagnostics of roller bearings in belt conveyors.

In cases where none of the discussed diagnostic methods are used, the state of production equipment (for example, the state of bearings in a belt conveyor) is usually monitored by sensory perception (visual control, auditive control of noise, tactile control of bearing box temperature, lubrication state, control of clearance, control of bearing rings slippage, etc.). These methods are arbitrary and highly unreliable.

Such an approach cannot yield satisfactory results because 1:

- The method of monitoring bearing state by visual examination requires production equipment operation to be halted, which can be done only when planned,

- It results in further existence of relatively high percent of delays due to breakdowns due to bearing damage.

Modern tendencies are to apply diagnostic methods and means in order to detect and determine causes of breakdowns. This is especially significant for those breakdowns which are considered unfounded yet occur with increasing frequency.

Table 2 describes research performed in order to determine the extent to which bearing breakdowns participate in the overall set of breakdowns, while Table 3 gives an overview of research performed by SKF and SPM (Shock Pulse Method) methods based on four causes of breakdowns in roller bearings.

Table 2.

Overview of the extent to which bearing breakdowns participate in the overall set of breakdowns.

| Analysis performed by | Paper Mill Quebec Canada | IEEE – electric motors (1141 el. mot.) | EPRI Study (6312 electric motors) | Average |
| --- | --- | --- | --- | --- |
| BEARING BREAKDOWNS | 44% | 44% | 41% | 43% |

Table 3.

Overview of breakdown research according to SKF and SPM methods.

| Method | Fatigue | Inadequate lubrication | Assembly | Other | Causes which can be influenced by maintenance |
| --- | --- | --- | --- | --- | --- |
| SKF | 34% | 36% | 16% | 14% | 66% |
| SPM | 11% | 44% | 25% | 20% | 89% |

It can be noted that both methods yield similar percentages of breakdowns resulting from inadequate lubrication and inadequate assembly.

Results favor the application of SPM diagnostic method for monitoring the state of roller bearings in a succesfull way.

Shock pulses in roller bearings result from impacts between rollers and roller tracks. Relationship exists between the state of the bearing and the size of the shock pulse.

As a diagnostic method, SPM method makes it possible to achieve progress in the field of evaluation of bearing state as well as maintenance activities. General experience however shows that it is necessary to move a step further because it is very difficult to evaluate the state of a bearing based on a single measurement. On the other hand, bearing manufacturers themselves emphasize the fact that instrument used for this purpose points out to good, worsened or bad operating state of a bearing. Such a state is not necessarily caused by bearing damage, impure lubricant, lack of lubrication, poor assembly, design errors, etc.

Statistics show that 2/3 of all roller bearing breakdowns result from inadequate or insufficient lubrication. Lubrication is an especially important issue because of the conditions production equipment operates in.

The aim of integrating the SPM diagnostic method into state-dependent maintenance as one of preventive maintenance methods is to promote a more organized functioning of the diagnostics department. The final objective of a well organized department with clear and detailed project for roller bearing diagnostics according to the SPM method is to determine:

- The number of bearings, their location, identification, priorities and grouping, suitability for measurement of all bearings, activities needed to select suitable control points where diagnostic measurements are to be carried out,

- Necessary equipment and means for performance of activities defined by the project,

- Dynamic schedule of planned activities,

- Necessary personnel and their structure, preparation and training,

- Preparation of documents for data collection, processing and archiving,

- Lubrication schedule, so that lubrication precedes measuring.

Whether any design, including the design of a belt conveyor, is suited for efficient application of SPM diagnostic method for the control of the state of roller bearings is a question susceptible to analysis. For example, each control point should be analyzed from the standpoint of accessibility for efficient maesurements.

Those control points which have been classified as unsuitable should be adapted to the appropriate level of suitability for use. Several possibilities exist, and some of them are:

- Protection from dirt deposits,

- Clearing the approach to the control point,

- Placement of measuring devices and

- Mounting of fixed sensors etc.

Fixed sensors can be used to place monitoring and control units connected at a single point. Cable connection is established between sensors and signals. The signal which points out that a breakdown has occurred can be audio or visual (good or bad operating state of a bearing, functional diagnostics). Operation of the technical system can be halted automatically as the signal is given or manually by a supervisor.

Decisions about necessary interventions are made by monitoring the state of roller bearings (by measuring and keeping record of data).

It is obvious that consistent application of SPM without improvisations provides significant protection from breakdowns on technical systems. This, in turn, causes positive economic effects which are not subject of this paper.

SPM diagnostic method for roller bearings in conveyors and for other production equipment should be used for state-dependent maintenance according to developed dynamics and especially selected, prepared and marked measuring control points.

5. INVESTMENTS INTO PRODUCTION EQUIPMENT

Previous discussions emphasize the significance of production equipment diagnostics during its exploitation. This is an important indication used in making decisions about the need to invest in order to reconstruct or modernize the existing activity. Even though the basic responsibility of the maintenance staff (which performs diagnostic activities) is to ensure regular and uninterrupted operation of equipment, they usually do not participate in this important process.

Even the current investment methodology does not give maintenance staff any influence on this process. The involvement of maintenance personnel into decision process related to investments into purchase of necessary equipment should, because of its significance, become compulsory and, as such, it should be based on the terotechnological approach 5.

Since proper functioning of production equipment (uninterrupted planned production during the economic life of the investment) can be achieved only by its continuous maintenance, this means that equipment for material production and its maintenance are directly dependent. Because of this fact, it is completely logical, justified and necessary to include skilled maintenance personnel in the process of investing - equipment purchasing.

The need for terotechnological approach to the process of analysing problems from the investment field appears in the development phase (design and construction) and as such it represents a direct continuation of research based on the activities discussed earlier in this paper.

Investing into necessary equipment requires its proper selection and appropriate classification suited to the specifics of each investment project. The selection of equipment necessary for the realization of certain products intended for the market depends on type of production, type of business activity and its complexity and is directly related to the workpost and space necessary for its arrangement and uninterrupted flow of planned production activities. Equipment should be classified in such a manner that it includes all necessary pieces, which means that the list should comprise all equipment necessary for the uninterrupted flow of planned production – Table 4 5.

Table 4.

Inventory of necessary equipment.

| # Equipment type | ## Operation | Equipment technical capacity | Number of pieces | Shift workers | ## Total |
| --- | --- | --- | --- | --- | --- |
| Production (technological) |  |  |  |  |  |
| Transport |  |  |  |  |  |
| Storage |  |  |  |  |  |
| Quality control |  |  |  |  |  |
| Maintenance |  |  |  |  |  |
| Power production |  |  |  |  |  |
| Environmental and protection at work |  |  |  |  |  |
| Infrastructure |  |  |  |  |  |

The approach which allows maintenance personnel to directly influence equipment characteristics starting from design phase results in favorable climate and favorable technical conditions for more efficient exploitation of equipment during the production process.

In order to include maintenance personnel in the equipment design process and in the analysis of its technical characteristics shown in table 4, the following has to be known: the behavior of equipment in exploitation (diagnostics for the purpose of monitoring and analysis of weak spots and detection of breakdowns), as well as detailed knowledge of technological production process used for the production activity in question.

The realization of above mentioned activities related to diagnostic process and its influence on the investment process, which are obviously related to state-dependent maintenance as one of the preventive maintenance methods, should be entrusted to qualified maintenance personnel during the development phase, through design and construction.

6. MAINTENANCE PERSONNEL TRAINING

Production equipment maintenance based on state-dependent maintenance mainly engages personnel for the process of equipment monitoring and analysis during its exploitation related to its diagnostisc but does not include maintenance personnel in the process of making decisions about the need to invest.

Considering the tasks maintenance personnel are expected to perform based on the explained diagnostic concept and the influence they have on the process of making decisions about the need to invest, their training and improvement are of special significance.

In order for the maintenance personnel to be able to succesfully apply scientific and professional knowledge from the field of production equipment maintenance it is necessary to provide them with professional guidance as well as with general technical knowledge.This is the case in all types of industries and especially in those where work is continuous (process industry).

Maintenance personnel are required to possess highest professional training as well as inclination to research while, at the same time they should be versed in the complex field of terotechnology. This represents a limiting factor and maintenance personnel should be promptly guided towards the fulfillment of these aims from the very beginning of their professional career 6.

Staff at the higher organizational level (maintenence managers and work preparation managers) are not adequately included in the decision making process aimed at the selection of equipment which is to be the object of maintenance activities.

Maintenance personnel does not usually exert much influence on the production equipment exploitation regime. In practice, the role of maintenance personnel is usually reduced to cooperation with production personnel in relation to the occurrence of delays and breakdowns, which should be eliminated. Maintenance personnel does not exert any direct influence on this process which is intollerable in modern production.

This practice puts maintenance personnel as well as maintenance function down when compared to personnel engaged in material production, causes discontent which is often manifested through negative effects and directly influences the product which is the final objective of production.

In any case, in order to equalize the significance of material production and maintenance and in order to establish close cooperation between these two activities it is necessary not only to include maintenance personnel into exploitation process but also in the process of decision making related to purchase of new equipment during the investment phase.

The influence of maintenance personnel on the diagnostic process, i.e. on the maintenance of production equipment, results in its increased reliability and decreased need to invest before the economic life of the product has expired.

In cases where breakdowns do eventually cause delays in the production process, which is sometimes inevitable regardless of the way used to organize planned diagnostic activities, maintenance personnel has to be trained for emergency actions in the maintenance process as well as for elimination of unexpected delays on different pieces of equipment. This means that, apart from specialized knowledge, they should also possess general technical knowledge.

Maintenance personnel is trained both theoretically and practically. This knowledge must regularly be updated and refreshed by attending various seminars, courses, professional conventions etc. 7.

As modern maintenance requires direct involvement of staff into the decision making process related to production equipment according to the terotechnological principle, its education and training represents the axiom of organized production.

This calls for close cooperation between equipment manufacturers and its users together with engagement of educated and qualified maintenance staff. This is in the interest of both material production workers and maintenance staff.

7. CONCLUSION

Production equipment diagnostics whose aim is to detect weak spots and causes of possible breakdowns is especially important because it is closely related to making decisions about the need to invest. Production equipment diagnostics is performed on those elements of production equipment which are expected to be »weak spots« and it is closely connected to detection and detailed analysis of their causes, because elimination of breakdowns significantly decreases delays. This has a positive effect on the efficiency of maintenance.

Production equipment diagnostics is performed during its exploitation and it is based on the results obtained by state-dependent maintenance activities.

Correct diagnostics of production equipment during its exploitation represents one of important factors for making decisions about the need to invest in order to achieve reconstruction and modernization of the existing activity.

Activities related to diagnostics and investment process which are obviously functions of state dependent maintenance as one of preventive maintenance methods should be entrusted to well trained maintenance workers starting from the very phase of development through design and construction. Maintenance workers represent a very important link in the decision making chain because they are responsible for regular and uninterrupted operation of equipment which is the subject of investments. All involved activities are based on the need to use the terotechnological approach in the process of analyzing the area of state dependent maintenance which also includes the investment process which is an inseparable part of a certain activity.

This method of work inevitably results in more responsible maintenance activities and maintenance staff as well as in decrease of the need to invest in production equipment before the expiration of economic life of a project.

The achievement of set objectives requires suitably educated maintenance staff and their direct involvement into the decision making process related to production equipment according to the principles of terotechnology. The approach based on state-dependent maintenance represents the very axiom of organized production and includes the necessity of cooperation between the manufacturer of equipment and its user which, in turn, calls for the engagement of well trained and versatile maintenance staff. This is in the best interest of both production workers and maintenance workers.

REFERENCES

- Grujic N. Development of Methods for Diagnosing Operational Functionality of Technical Systems, (in Serbian), Technical College, Pozarevac, 1998.

- Soldat D. Efficiency of Maintenance, (in Serbian), KIZ “Altera”, Beograd, 1993.

- Grujic N. Analysis of Mechanical Systems from the Standpoint of Diagnosing the Operational State (in Serbian), Masters thesis, Faculty of mechanical engineering, University of Nis, Nis, 1989.

- Wohlebe H. Tehnische diagnostik im maschinenbau, (in Germany), Dresden, 1997.

- Plavsic R., Soldat D., Dubonjic R., Milanovic D. Lj., Knezevic S. Preparation of the Investment Study, (in Serbian), A.S. “Delo”, Beograd, 1998.

- Tucakov M. Maintenance Personnel, (in Serbian), Journal of Terotechnology, 18(1989)4, 215.

- Plavsic R., Soldat D. Workspace, Equipment and Personnel in Production Maintenance Process, (in Serbian), Tehnika – Organizacija rada, 1996, vol.46, 1-2, pp.4-8.

- Поступила в редакцию 10 августа 2001 года.

Economic life of a project relates to the period of planned investment realization considering the market and technological obsoleteness of equipment usually during a ten year period.