ONE ALGORITHM OF THE SOLUTION OF THE FRACTURE MECHANICS PROBLEMS FOR THE FINITE ELASTIC BODIES

Leontiev A., Tuchkova N.*, Lurie S.**

Institute of Mathematic, Federal University of Rio de Janeiro, Rio de Janeiro, Brazil

*Dorodnicyn Computing Center of RAS, Moscow, Russia

**Institute of Applied Mechanics of RAS, Moscow, Russia

SUMMARY

The fracture problem for finite elastic body under static loading with unit crack or set of internal and surface cracks is considered. Using special orthogonal expansions, an approximation method to define the stress intensity factors is proposed. The orthogonal expansions in singular and regular parts are constructed in the norm associated with the potential energy. The proposed method allows to find the approximate values of the stress intensity factors for 2D and 3D problems. The asymptotic singular solutions near the tops of cracks, potential energy and Lagrangian for considered elasticity problem must be obtained in order to define the stress intensity factors.

## INTRODUCTION

The main problem of the fracture mechanics consists in the determination of the stress intensity factor. This problem can be subdivided into two stages. In the first one, the asymptotic singular solution, which ensures the continuity of the displacements at the singular point and the boundedness of the strain energy, is obtained. The local problem of the theory of elasticity is solved in the neighborhood of the crack top. This problem presents no special difficulties. The asymptotic singular solution for the orthotropic body can be constructed, for example, by using Lehnickii’s potentials [1] and conversion of Mellin’s transformation. The construction of the asymptotic singular solutions for the three-dimensional elastic body represents certain complexity. Some singular solutions for 3D problem were obtained in [2]. The second stage consists in the determination of the amplitude in the singular solution (coefficient of the stress intensity factor). This part presents the main difficulty, since requires solution of the problem as a whole.

#### 1. ASYMPTOTICAL SINGULAR SOLUTIONS

We consider the linear problem for the elastic body. The model of material is specified by the potential energy. We assume that singular solution has the power singularity for the stresses in neighborhood of some singular points or lines. In the general case the singular solution can be written in the form Rs=rβlnrδW(r,ϕ,r0,α), where W(r,ϕ,r0,α)is a regular function; r0=(x0,y0) is the translation vector; α is the angle of the rotational transformation; ψ(α)=exp(iα) defines the position of the singularity with respect to the global coordinate system; r,ϕ are the local polar coordinates in vicinity of the singularity; β,δ are constants.

As an example we consider a two-dimensional body with the unit crack and give the singular solution near the end of the crack for harmonic and biharmonic problems. Boundary and volume external loading is assumed. The edges of crack are free from the loading. We consider first the harmonic with respect to displacement function v(x,y) problem:

∂2v∂x2+a2∂2v∂y2=0, 									(1)

where a2 is a real parameter. We would like to write the asymptotic singular solution of the equation (1) in polar coordinates r, ϕ related to the top of crack. This solution vs(x,y) satisfies the following boundary conditions on the edges of crack:

∂v∂y=0 for y=0, ϕ=0,

and		∂v∂y=0 for y=0, ϕ=2π.							(2)

The edge x>0, y=0, ϕ=0 is the upper edge of the crack and the edge x>0, y=0, ϕ=2π is the down edge of the crack. Singular solution of the problem (1)–(2) is written to within the amplitude As:

vs=As|z|12cos(ϕ2).								(3)

Here z=x+i(a−1)y, r=z=[x2+(a−1)y2]12 and i=−1.

Function vs is continuous at the singular point x=0,y=0. The derivatives of this solution have power singularity in the singular point:

∂ vs∂ x=As12|z|−12cos(ϕ2),

∂ vs∂ y=As12(a)−1|z|−12sin(ϕ2).							(4)

Note, that this problem models the crack of the normal opening crack for the material with normal deformation along axis x equal to zero. Relations (4) define the distributions of the normal and shear stresses in vicinity of the singular point.

Let us consider now the biharmonic problem for orthotropic elastic body with crack. The singular solution can be written using stress function ϕ to within the amplitudes A11,A12, since in the general case the normal opening crack is not divorced from the crack of transversal shear:

σx=2Re[μ12F1'(z1)+μ22F2'(z2)], σy=2Re[F1'(z1)+F2'(z2)], τxy=2Re[μ1F1'(z1)+μ2F2'(z2)],							(5)

Exu=2Re[p1F1(z1)+p2F2(z2)], Eyv=2Re[q1F1(z1)+q2F2(z2)],

ϕ=(|z1|λ+1cos(λ+1)θ1−|z2|λ+1cos(λ+1)θ2)A11−−A12(|z1|λ+1sin(λ+1)θ1−|z2|λ+1sin(λ+1)θ2),

Fk(zk)=ϕk'(zk), ϕk'(zk)=dϕdzk, pk=μk2−νxy,qk=μkνxy−μk−1, zk2=x2+an2y2,

θk=arctg(−akyx), ak=−iμk,  z1=x−μ1y, z2=x−μ2y.

Hereσx,σy and τxyare the normal and the shear singular stresses, R1(x,y)≡u(x,y) and R2(x,y)≡v(x,y)are the components of displacement vector along axis x and y respectively; μ1 and μ2 are the roots of the algebraic equation: μ4+2pμ2+q=0, p=EGxy−νxy , q=ExEy,  Ex, Ey, Gxy are the elasticity modulus of the orthotropic body, νxy is the Poisson coefficient, Exμxy=Eyμyx.

Note, that solutions (3) and (5) are written in the local polar coordinates related to the tops of the cracks along the axis x. In the general case the location of the cracks with respect to some global coordinate system are defined by the translation vector r0=(x0,y0) and rotational transformation ψ(α)=exp(iα), where angle α gives the crack orientation.

##### 2. ALGORITHM OF DETERMINATION OF STRESS INTENSITY FACTOR. APPROXIMATE SOLUTION OF THE PROBLEM

We consider the linear elastic problem in the variation form:

δL=δ(U−A), U =12VCiα jβ AiαAjβ dV,						(6)

A=VXiRi dV +Ft=1sYiβs−t Ai βs-t(−1)t+1 dF.

Here Aij=∂Ri∂xj, Ai 0=Ri, Ai kp =∂ Ri∂ xk∂xp, Ai βt=∇r1∇r2....∇rtRi; multi-index αs (of the order s) denotes that tensorAiαsis formed by means of operator ∇r1∇r2..∇rss on the vector Ri; tensor Ciαjβis the symmetric tensor with respect to the combination of indexes  iα and jβ:  Ci α  j β=Cj β i α, M=rang C; M=2(s+1); α and β are the multi-indexes of orders;Aiαk, Aiβt are the tensors of k+1and t+1 order; 1≤k≤s, 1≤t≤s respectively; Aiαare the tensor’s arguments of the density of deformation energy: Ai k =∂ Ri∂ xi∂xk≡∂ Ri∂ r1∂r2, r1=xi, r2=xk and so on.

The Lagrangian density L in the equation (6) depends not only on vectorRi, strain tensorAij=∂R∂xj, but also on the curvature tensors of higher order of Aiαs. It means that the energy density depends on the generalized bending or, in the other words, on the twisting volume element.

The algorithm to define the stress intensity factors is based on the procedure of orthogonal decomposition of the solution. This decomposition is performed into orthogonal subspaces of the singular and regular solutions in the norm associated with the strain energy.  With this procedure the solution is constructed in the form of direct sum of orthogonal subspaces. The expressions for the potential energy do not contain cross terms. The Lagrangian of the problem is written in the additive form.

Definition 1. The coefficients in the orthogonal decomposition of the solution to singular and regular parts are proposed to treat as the stress intensity factors.

Suppose that the set of vector-functions M defines the subspace of sufficiently smooth solutions of the boundary-value problem corresponding to the variation equality (6).  As the norm we take the quadratic form coinciding with the strain energy:

R2=R,R=2U, 2U=VCiαjβ∇αs(Ri)∇β(Rj)dV,

where ∇αs=1s∇rk(), Ri are the components of vector- function R∈M.

The scalar product of two arbitrary elements ℜm∈M and ℜn∈M is defined as:

[ℜn,ℜm]=  VCi α j β∇α(ℜjn)∇β(ℜjm)dV=2Unm.

Consider some finite system of the vector-functions Rj, Rj∈M, j=1,2,...J with componentsRij. The set of the vector-functions Rjj=1j=J determines the subspace ΜJ∈M. The following theorem is valid:

Theorem 1.

1. For any vector-function R from M and the given system of the linearly independent vector-functionsRjj=1j=J, which determines subspaceΜJ, can be obtained the following orthogonal decomposition:

Ri=Ri+Ri.										(7)

Here R is the projection of vector Ron the subspace ΜJ, and vector-function Ri is determined by the equation: Ri=Ri−RinUnm2[Ri,Rim]=Ri−bkRik, where Un k Uk m= δn m andδn m is Kronecker's delta, Uk m=Rk,Rm.

2. Any vector-function Rm∈MJ is orthogonal to the vector-function Ri within the integral norm: R2=R,R=2U.

3. Vector-function R is determined by the known functions Rk:

Ri=bkRik,   where  bk=AmUm n2,						(8)

Ak=A(Rk)=VXjRjk+FYj βs-tAj βs-tk(−1)t+1dF.

The proof of theorem is given in [3], [4]. The theorem specifies the structure of the orthogonal decomposition and gives formulas (8) for the coefficients bk of this decomposition. Let us assume that ℜ(..) is the resolving operator for the problem (6), and ℑp(..) are the boundary operators of the boundary-value problem (6).

Theorem 2.

1. The generalized volumetric forces and generalized surface forces in the functional (6) are presented in the form of expansions with the same constants bk as in the expansions (8):

Xi=Xi+bkXik,	Yi βp=Yi βp+bkYi βpk,					(9)

whereXikandYi βpkare defined by the given set of functionsRik: Xik=−ℜ(Rik), Yi βpk=ℑp(Rik), p=s−t, t=1,s; functions Xi and Yi βp can be defined as following: Xi=−ℜ(Ri),  Yi βp=ℑp(Ri),p=s−t, t=1,s.

2. Variation statement of the boundary-value problem for vector-functions R of the subspace orthogonal to ΜJis given by the equality (6). In this equation ”work” A(X,Y βp) of the given forces X,Y βpmust be replaced by the ”work” A=A(X, Yβp) of the generalized forces X, Yβp which can be found from equations (9). Consequently, the boundary-value problem for the vector-functionsRi is completely determined by the generalized forces Xi, Yi βp.

3.  The following equations have place:

VXiRki+FYj βs-t[∇ βs-t(Rjk)] dF =0,  for  all Rik⊥Ri.

VXki Ri+FYjkβs-t[∇ βs-t(Ri)] dF =0, for all Rik⊥Ri.			(10)

Proof of the theorem is obvious, see [3].

The relationships (10) allow to interpret the orthogonality of "kinematic states" Rik and Ri. These vector-functions are orthogonal in the sense that the "work of generalized forces" Xi and Yj βs-t on the kinematic states Rik is equal to zero.  On the other hand, the "work of generalized forces"Xki and Yjk βs-ton the kinematic state Ri is equal to zero also. As a result, the algorithm of the determination of stress intensity factors is reduced to the calculation of the work of singular solutions on the generalized "loads" (in the volume of body and on its boundaries), and to the calculation of strain energies for the singular components.

Let us assume now that the set of vector-functions of the singular solutions Rij≡Risj in the orthogonal decomposition (7) is found.

Definition 2. Suppose that subspace R is given in the form of expansion in terms of the set of functions orthogonal to each vector-function from the system of singular solutions Risj. Then we say that the exact decomposition of the regular and singular solution is obtained and the coefficients of intensity are found accurately.

Really, in this case the subspace R does not contain the «traces» of the singular solutions. If the requirement of Definition 2 is not carried out, then we say that obtained solution is an approximate solution of the stress intensity factors problem. Let us note that in [5] is used the expansion (8) to construct the approximate solution by variational methods as first approximation. The second approximation is constructed using the orthogonal subspace R, etc. With this procedure for the second approximation the traces of the first approximation will remain only in the boundary conditions. It is not difficult to see that in the case when decomposition (8) is constructed on the singular solutions with the power singularities, the traces of the singular solutions for the second approximation will tend to zero if the size of region with the crack grows and the distance from the top of crack to the boundaries of the region tends to infinity. So, it is possible to assume that for sufficiently large regions the expansion (8) gives practically exact solution.

Let us note that this solution is analytical and is sufficiently convenient for the system of the closely spaced cracks. In this situation the mutual influence of singularities is determined by the mainly appropriate asymptotic behaviors of the singular solutions.

In the case of single crack the non-standardized stress intensity factor can be calculated by the formula (8) using known asymptotic behavior of the singular solutionRs:

bk=As2U[Rs,Rs], As=A(Rs).							(11)

Here R1≡Rs, Asis the work of singular solution Rs on the given forces

distributed in the volume of the elastic body and on its surface, U[Rs,Rs] is the potential energy of the singular deformation state.

### Let us consider the finite plane body with a finite number of cracks. The edges of cracks can be loaded or free of loading. Since each surface crack is connected with one singular point and internal crack is connected with two singular points, we have a finite number of singular points. Let us define for each singular point the singular kinematic state Rsj(x,y):

Rsj:Risj(x,y)=fij(x,y,xj0,yj0,αj)|zj|λj,

where Risj is the asymptotic of the singular energy solutions, j is a number of singular points, xj0,yj0 are the coordinates of these points, αj is the orientation angles for each of singularities. The parameters λj also can be distinct and depend on the type of singularity at each singular point. In particular, if all singular points in two-dimensional isotropic problem represent the ends of cracks, then all parameters λj are equal. From other side, the sources of singularities can be the corner points (conical points). Taking into account Theorem 1 and Definition 1, the non-standardized stress intensity factors can be found as a solution of the following system of the linear algebraic equations:

[Risj,Risk]bj=Ask,  Ask=A(Rks)   or   bsj=UsjmAsm,				(12)

where UsjmUspk=δpjδmk, 2Uspk=[Risp,Risk],  Askare known. For example for the classical elasticity model we haveAsk=VXiRiskdV+ΓYiRiskdF, where Xi and Yi are given volume and surface forces, respectively. In fact, stress intensity factors for the finite number of internal and surface cracks can be found in the explicit form as solutions of finite-dimensional linear algebraic system of equations (12) within the framework of first approximation.

As an example let us write down approximate solution of the problem for the normal opening crack in the orthotropic body. The crack is oriented along the axis ox, the top of crack coincides with the origin of the coordinate system. We use the singular solution (5).  Note that in the general case the mode of the normal opening crack is not separated from the mode of the transversal shear crack. For the case of the normal opening crack the component of displacements u(x,y)≡R1(x,y) along the axis x must be even function of the coordinate y, and the component of displacements v(x,y)≡R2(x,y) along the axis of transversal coordinate must be odd function ofy. In particular, for the normal opening crack we obtain from asymptotic singular solution (5) the following expressions for the components of the displacements:

R1s=|z1|12p1cosθ12−|z2|12p2cosθ22	R2s(1)=|z1|12q1sinθ12−|z2|12q2sinθ22 .

Then, the approximate value of the stress intensity factor for the surface crack of normal opening in the orthotropic plane body of arbitrary form is given by the following equation:

bs=Ω[X1R1s+X2R2s]dxdy+Γ[Y1(s)R1s+Y2(s)R2s]i(s)dsΩa11∂R1s∂x2+a22∂R2s∂y2+2a12∂R1s∂x∂R2s∂y+b∂R1s∂x+∂R2s∂y2dxdy,

where bs=A11, Ωis  the body area, Γis surface profile of body Ω, s is contour coordinate.

Let us consider now the plane with finite internal crackx,y; x∈[−1,1], y=0 loaded by the normal force g(x). Body distributed forces and stresses on infinity are equal to zero. Classical accurate result lies in the fact that the stress intensity factor is determined by the value: −11g(ξ)a+ξa−ξdξ. On the crack x∈(−a,a),y=0) the expression a+ξa−ξ represents the trace of the transversal component of the displacement vector v(x,y) for the appropriate singular solution (see [6]). This solution is written in the form:

u≈[(1−2v)Re(z+a+z−az−a)−yIm(−1(z−a)z2−a2)],

v≈[2(1−v)Im(z+a+z−az−a)−yRe(−1(z−a)z2−a2)],

where v is the transversal component of the displacement vectorRs, u is its longitudinal component. Our algorithm determines exactly the same expression for the stress intensity factor. Consequently, for the infinite plane, approximate solution coincides with the exact solution.

# 3. REFINEMENT OF THE STRESS INTENSITY FACTORS

As it was noted earlier, in the general case the approximate solution of the problem is given by formulas (11) and (12). The algorithm of the refinement of the stress intensity factor for single singularity follows directly from the next theorem:

Theorem 3.

Assume that vector-function Rs0 with the components Ris0 is the singular displacement in the neighborhood of the singular point. Assume also that system Zik(x,y) of vector-functions satisfies all conditions, which are necessary to find any approximate solution in the form of expansion within the framework of variation procedure. Then solution of the problem of the determination of stress intensity factor for the single singularity is given by the formulas:

Ks=bs−ak[Zik,Ris0][Ris0,Ris0],								(13)

where coefficient bs is found from equation (11) and a are solutions of the following systems of the linear algebraic equations:

aUkm=Asm,										(14)

with 		A=A(X,Yβp), Asm=A(Zism),  Zsik=Zik−R0si[Zik,R0si][R0si,R0si],

Ask=VXiZsik dV +Ft=1sYiβs−t Aik βs-t(−1)t+1 dF  , Aikβt=∇r1∇r2....∇rt(Zsik),

Xi=Xi,Ysi βp=ℑp(Rsi0),	Yi βp=Yi βp−bsYsi βp.

In the classical theory of elasticity the system of the algebraic equations (14) can be written in the following form:

ak[Zisk,Zism]=VXiZismdV+ΓYiZismds,

here Xi=Xi,  Yi=Yi−bsYsi,  Ysi=Cptijnj∂Rsp0∂xt, nj is normal to the surface on the body, Cαβij is tensor of elasticity modulus.

To prove the Theorem 3 let us write the solution as a sum of singular and regular parts:

Ri=KsRsi+kakZik=bsRsi+kakZsik.

The proof follows from the orthogonality of the vectors Rs and Zsk. In fact, to find the stress intensity factor we must find the coefficient bsusing equation (11). At the second stage using the variation procedure we must construct the solution in the subspace orthogonal to vector Rs.

As an example, we consider plane harmonic problem for finite and symmetric with respect to axis y body G. The body is loaded by distributed force X(x,y). The boundary of the body is loaded by force Y(s), s is the contour coordinate on the boundary. Assume that the boundary problem is symmetric with respect to axis yas a whole. So, we consider the harmonic problem with Neumann’s boundary conditions. Assume that necessary conditions for the existence of solution are fulfilled. Taking into account equation (3) we put vs=Rs0(x,y)=|z|12cos(ϕ2). Assume that in the equation (1) a=1 and the body lies inside of the rectangle 1≤x≤−1, 1≤y≤−1. As the set of functions Zk (see Theorem 3) we can take complete system of functions:

Zk(x,y)=[cos(πky);sin(π(2k−1)x2; cos(πkx)].

Complete solution of the problem of the determination of the stress intensity factor is given by the formula:

Ks=bs−ak[Zk,Rs0][Rs0,Rs0]	Zk=[cos(πky);sin(π(2k−1)x2; cos(πkx)],

where  	bk=GXRs0dxdy+ΓYRs0dsG∂Rs0∂x2+∂Rs0∂y2dxdy, Rs0(x,y)=|z|12cos(ϕ2).

The parameters akare found from the system of the algebraic equations:

ak[Zsk,Zsm]=VXZsmdV+ΓYZsmds,

Zsk=Zk−Rs0[Zk,Rs0][Rs0,Rs0],  X=X, Y=Y−bs∂Rs0∂n|Г,

2U[Zsi,Zsj]≡[Zsj,Zsj]=∂Zsj∂x∂Zsj∂x+∂Zsj∂y∂Zsj∂y dxdy.

For the rectangular region G it is possible to show that the solution of system ak[Zsk,Zsm]=VXZsmdV+ΓYZsmds can be found in the form of exact expression despite the fact that the functions are not orthogonal in the norm R2=R,R=2U. Consequently, the solution of problem as a whole can be found in the form of exact expression. All these results could be extended to the general case of asymmetrical problem.

At last, we must note that the generalization of the Theorem 3 is valid in the general case of finite numbers of cracks (finite number of singular points). Assume that we solved the approximate problem for finite number of cracks using formulas (12). We can find the approximate singular part of the solution

Rsi0=1NbskRsik(x,y), where Rsij(x,y)=fij(x,y,xj0,yj0,αj)|zj|λj

and N is the number of singular points. Using the set of the finite number of vector-functions Rsij and standard orthogonalization procedure we can obtain new system of functions ψsij, which are orthogonal in the norm R2=R,R=2U. Then we can rewrite the relation for approximate singular part of solution in the following form:

Rsi0=1Ndskψsik(x,y),

where dsk is a known constant obtained, due to orthogonalization procedure for functionsRsij, as a linear combination of the constants bsk. The following theorem describes the algorithm of refinement of the coefficients dsk.

Theorem 4.

Assume that vector-function system Zik(x,y) satisfies all conditions, which are necessary to find any approximate solution in the form of expansion within the framework of variation procedure.  Then solution of the fracture mechanics problem for the finite number of singularities is given by the formulas:

Ks=dsj−ak[Zik,ψsij][ψsij,ψsij],								(15)

where coefficients dsj are found from the equation (12) and a are solutions of the following systems of the linear algebraic equations:

aUkm=Asm,										(16)

with		A=A(X,Y βp), Am=A(Zism),  Zsik=Zik−j=1Nψsij[Zik,ψsij][ψsij,ψsij],

Ask=VXiZsik dV +Ft=1sYiβs−t Aik βs-t(−1)t+1 dF  , Aikβt=∇r1∇r2....∇rt(Zsik)

and

Xi=Xi,  Yi βp=Yi βp−Ysi βp,

Ysi βp=ℑp(Rsi0) , Rsi0=1Ndskψsik(x,y).

Theorem 4 is the generalization of Theorem 3. This theorem makes possible to refine coefficients dsj and, consequently, to refine the stress intensity factors bsk. In accordance with the theorem, it is necessary to determine the constantsa preliminarily. For this purpose an auxiliary problem in the subspace orthogonal to Rsi0 should be solved by the variation method. Solution of this auxiliary problem must be obtained in the form of expansion on the special set of functionsZsik and is equivalent to the solution of (16).

# CONCLUSIONS

In this paper the algorithm of calculation of the stress intensity factors for elastic bodies with finite number of arbitrary-oriented singularities (surface and internal cracks, corner points) is proposed. The determination of the stress intensity factor is very important point to study the resistance of the materials to bursting and destruction. The proposed algorithm is sufficiently general. This algorithm is suitable for the numerical implementation and, from this viewpoint, can be applied to treat the optimization problem in the fracture mechanics. It is applicable both to two- and three-dimensional problems of the classical theory of elasticity and also to the analogous problems of the fracture mechanics with more complicated models of deformation provided that the asymptotic singular solutions can be constructed.

REFERENCES

- Lehnickii’s S.G. Theory of the elasticity of anisotropic body. Moscow, 1950.

- Fedoruk M.V. Asymptotic behavior of the solution of Dirichlet problem for the equation of Laplace and Helmholtz in the appearance of the thin cylinder. Izv. AN SSSR, Mathematical Ser., 1981, v.45, p.167-186.

- Lurie S.A., Belov P.A. Mathematical models of the solids mechanics and physical fields. Moscow, Computing Center RAS, 2000, p.151.

- Tuchkova N.P. On the determination of the solution projections of boundary-value problems on the given linear subspace of the functions. Moscow, Computing Center RAS, 2003, p.32.

- Morozov N.F. Special two-dimensional tasks of the theory of elasticity. L., 1980.

- Tuchkova N.P. Orthogonal kinematics expansions in variational method of the boundary-value problems for the mechanics of solids bodies, Dissertation, Institute of Applied Mechanics RAS, M., 2004.

- Поступила в редакцию 19 июля 2004 года.

This work was financially supported by the CNPq (Conselho Nacional de Desenvolvimento Científico e Tecnológico, Brasil), grants 305233/2003-3 and 450066/2004-5 and Russian Foundation for the Basic Research, grant N 03-01-00165.